2011

nklee

ServnTec Co., Ltd.

2011-01-15

HD Hybrid Install Manual

HD Hybrid System

Contents

1	Before you proceed	4

2	Version Management	5

3	DVR Hardware specification	6

4	HOW TO INSTALL	9

4.1	HDH-4820, HDH2412 Model	9

4.2	HDS-2408, HDS-1204 Model	11

4.3	SDH-H4816L Model	12

4.4	HDH series & SDH Series Set (except for SDH-H4816L )	13

4.5	SDH-H4816L Set	14

5	When installing DVR board : location of DVR board on mother Board slot	15

5.1	DVR board installation & connecting cable	15

6	OS / software install	17

6.1	OS install	17

7	Mother board & VGA driver Install	18

7.1	Setting Display Resolution	18

8	Screen Saver & Power Option	19

8.1	Screen Saver OFF	19

8.2	Setting Power option	19

8.3	Manual backup	20

8.4	“OFF” of Keyboard power and sleep button	20

9	HDD Partition & Format	21

10	DVR Driver & Program Install	22

11	Operating DVR	24

Appendix 1. Bios Setup	25

Appendix 2. Cautions when selecting Video Coaxial Cable	27

Appendix 4. Cautions when selecting BNC	31

Appendix 5. Cautions wehn installing HD-SDI Cable	34

- Before you proceed

This manual is to explain how to install HD Hybrid Series. The users who never installed before are recommended to read all the details in this manual beforehand.

The manual can be updated in order to improve the specifications and functions without previous notices.

<Notices>

- Check the Mother Board compatible lists before selecting PC Hardware.

- Install mother board and VGA board driver after installing OS.

After installation, you should connect the camera before powering on the system.

- The recommended cable is 75ohm and use cables under 20db per 100m or 6db per 100ft in 1GHz.

- BNC Connector should be 75ohm. If you use other ohm connectors it could damage on the circuit due to reflection.

- You should use Tool kit offered by connector companies when making BNC cable and make it according to the specifications.

- HD-SDI cable supports Single link. Therefore, you may not cut or extend the cable. To do so, you need extra devices. (HD-SDI Repeater, distributor and etc…).

- Version Management

| - Date | - Version | - Contents |
| --- | --- | --- |
| - 2011-01-31 | - Rev 1.0 | - Manual for initial version |

- DVR Hardware specification

| - List | - Specification | - Note |
| --- | --- | --- |
| - Mother - Board | - More than PCI Express X4 Slot - (except for VGA Slot) - Intel PCI controller(P55, H55, etc) | - Refer to Mother board Compatible Lists |
| - CPU | - More than Intel I5 - (HDS2408 model- more than I7) |  |
| - Memory | - More than 2GB |  |
| - VGA | - PCI Express x16 - Over Video memory 256MB | - Recommend items from ATI company |
| - Power | - Over 500W |  |
| - OS | - Windows XP, Windows Vista, Windows7 - 32bit OS | - - Supporting 64 bit in the near future - - OS : windows 7 |
| - Monitor | - Recommend monitor supporting 16:9 |  |
| - Resolution | - 1280x720, 1920x1080 | - GUI is automatically regulated up to the resolution |

Composition of Products

| Model |  | Board image | Channel | Frame (fps) | Note |
| --- | --- | --- | --- | --- | --- |
| HDH-4820 | HD-SDI |  | 4 | 120 (720P) 72(1080P) |  |
|  | H.264 (D1) |  | 16 | 480(D1) |  |
|  | Sensor (EIO Series) |  | I/O 4/4 |  | Option |
| HDH-2412 | HD-SDI |  | 4 | 120 (720P) 72(1080P |  |
|  | H.264 (D1) |  | 8 | 240(D1) |  |
|  | Sensor (EIO Series) |  | I/O 4/4 |  | Option |
| HDS-2408 | HD-SDI |  | 8 | 240 (720P) 144(1080P) |  |
|  | Sensor |  | I/O 8/8 |  | Option |
|  | P/T/Z |  |  |  | Option |
| HDS-1204 | HD-SDI |  | 4 | 120 (720P) 72(1080P) |  |
|  | Sensor |  | I/O 8/8 |  | Option |
|  | P/T/Z |  |  |  | Option |

| Model |  | Board Image | Channel | Frame (fps) | Note |
| --- | --- | --- | --- | --- | --- |
| SDH-H4816 | H.264 (D1) |  | 16 | 480(D1) |  |
|  | Sensor (EIO Series) |  | I/O 4/4 |  | Option |
| SDH- H2408 | H.264 (D1) |  | 8 | 240(D1) |  |
|  | Sensor (EIO Series) |  | I/O 4/4 |  | Option |
| SDH-H4816L | H.264(D1) |  | 16 | 480(D1) |  |
|  | Sensor (EIO Series) |  | I/O 4/4 |  | Option |

- HOW TO INSTALL

- HDH-4820, HDH2412 Model

| Board | Cable | Cable name |
| --- | --- | --- |
|  |  | Serial SATA Cable |
|  |  | 4BNC, 4RCA Cable |
|  |  | Watch-dog cable |
|  |  | 20 pin box Cable |
|  |  | USB Cable |
|  |  | RS 485/422 Cable |

H.264 D1 board (for PCI)

- H.264 D1 board (PCI Express)

- HDS-2408, HDS-1204 Model

| Board | Cable image | Cable name |
| --- | --- | --- |
|  |  | Serial SATA Cable |
|  |  | 10Pin Box cable |
|  |  | FDD Cable |
|  |  | 10 Pin Box Cable(USB) |
|  |  | P/T/Z power Cable |

- SDH-H4816L Model

| Board | Cable | Cable name |
| --- | --- | --- |
|  |  | Serial SATA Cable |
|  |  | 16BNC, 1RCA Cable (Camera input) |
|  |  | 16RCA Cable (Audio input) |
|  |  | Watch-dog cable |
|  |  | 20 pin Box Cable |
|  |  | USB cable |
|  |  | RS485/422 Cable |

- HDH series & SDH Series Set (except for SDH-H4816L )

| Board | Cable image | Cable name |
| --- | --- | --- |
|  |  | Serial SATA Cable |
|  |  | 4BNC, 4RCA Cable |
|  |  | Watch-dog cable |
|  |  | 20 pin BOX Cable |
|  |  | USB Cable |
|  |  | RS485/422 Cable |
|  |  | 40 pin Box Cable |

- SDH-H4816L Set

| Board | Cable image | Cable name |
| --- | --- | --- |
|  |  | Serial SATA Cable |
|  |  | Watch-dog cable |
|  |  | 20 pin BOX Cable |
|  |  | USB Cable |
|  |  | RS485/422 Cable |
|  |  | 40 pin Box Cable |

- When installing DVR board : location of DVR board on mother board slot

<Note> You may use Express x1, Express x4, etc (less than 16 board) for PCI Express x16 Slot

- DVR Board installation and cable connection

<Note> HD-SDI board is installed in PCI express x16 slot. It is recommended to install as below to fix the board inside slot.

- OS / software install

<Order>

- OS Install

- Mother board & VGA Install

- DVR related driver & program Install

- Creating Image DB

- OS Install

- Status of Models

| OS | Model | Note |
| --- | --- | --- |
| X86 O/S(32bit) (Window XP) (Windows Vista) (Windows 7) | HDH4820, HDH2412, HDS2408, HDS1204 SDH-H4816, SDH-H2408 SDH-H4816L | 64 bit will support. (Recommended O/S is Windows 7) <note> Multi OS is not recommended. |
| X64 O/S(64bit) (Windows XP 64bit) (Windows Vista 64 bit) (Windows 7 64bit) | SDH-H4816, SDH-H2408 |  |

- Caution for OS installation

Users have to devide partition when installing OS because users can’t make image DB in C drive which OS is installed.

- Mother board & VGA driver Install

<mother board driver>                       <VGA driver>

You have to install driver offered by the board supplier.

<Note> Driver is installed automatically with plug and play supported by O/S.

But this is a compatible device for installation, not an exact driver technically.

- Setting Display Resolution

Control Panel =>Appearance and personalization=>Select Adjust Screen Resolution

<Note> There occurs distortion in GUI if portions are not fitted according to resolution.

Thus it is recommended to set 1280x720, 1920x1080 for 16:9 and set 1024x720, 1280x1024 for 4:3.

.

- Screen Saver & Power Option

- Screen Saver OFF

If windows is not operating for some time, screen saver operates. So turn off screen saver for surveillance.

.

<Note> Do not use Aero Themes function.  When using it, camera image is not displayed.

- Setup of Power option

- It is a setup to prevent the operation of Display OFF, Power on suspend, Power of Keyboard, Sleep key during the operation.

- Change of Power Setup

- Manual backup

- Keyboard switch, sleep button OFF

21

- HDD Partition & Format

- It sets partition for format in order to use the spare of HDD because windows sets partition only for operations. This partition is used for DVR storage.

<4. 매뉴얼 백업 프로그램을 이용 검색>

- DVR Driver & Program Install

By using Install CD, you can make Program & image DB for driver related to DVR.

,

Step 1: Driver Install – Select the model for installation

Step2: DVR program install – select the language

Step3:  Image DB Install

Select the driver for Image DB                         Set the portion as %

- Operating DVR

- Connect the camera in the status of power OFF.

<Note> It is recommended to power on DVR after connecting Camera

because HD-SDI B’D sets camera format automatically.

- “On” the power

- Check DVR operation

Check functions through users ma

users manual mmamanual

Appendix 1. Bios Setup

Appendix 1.1 C-State Disable

<Note> C-state is for power saving of CPU. (function for Cell phone long ago)

- Symptom: After a while of normal recording, the number of display & saving frame drops or the board is not read properly.

- Setup

- When butting, it enters BIOS setup.(when starting, puts ‘del’ key)

- <Note> It depends on boards, therefore, refer to the manual for mother board.

- The following is an example of Asus

- Select CPU configuration from “Advence” setup

- Select disabled in Intel ® C-Stat tech.

Appendix 1.2 Set Power On always

When setting power on after rebutting

- Select “Power” and “APM Configurition”

- Select “power On” from ‘restore on AC Power Loss”

Appendix 2. Cautions when selecting Video Coaxial Cable

HD-SDI signal uses higher frequency signal due to lots of data. Analog is less than 10MHZ but HD-SDI uses higher frequency 100 times higher.

Higher frequency means faster attenuation from shortening wave length. Therefore, it is important to lessen attenuation.

To do so, you need to check the below points when selecting cable.
You should check if Coaxial Cable’s Impedance is for 75ohm. Also, you need to check the cable- Attenuation should be within 6db(6~6.2db) per 100ft for 1GHz. Coaxial Cable is also for 75ohm of  Impedance.

- Attenuation

| - Frequency | - 100ft distance | - 100m distance |
| --- | --- | --- |
| - 1GHz | - Less than 6db | - Less than 20db |

Our SDI transmission distance will be 100m with a cable having the above conditions

2. Cable Impedance 
Impedance : 75 ohm
In general, Coaxial cable for analog is a 75ohm cable. Because attenuation is relatively less when using 75ohm, HD-SDI should use a 75ohm. For this reason, all of our products were set with 75ohm impedance for H/W.

<Note>

Even for the same type of cables, you should check the specifications of a cable offered by cable manufacturer because the quality is different depending on the manufacturer.

Among analog Video Cable RG series, the following cases as below might have the higher attenuation. Therefore you may not use those ones.

World-widely well-known cable is Belden 7731A Coaxial Cable from Belden

Appendix 4. Cautions for selecting BNC

Canare BNC connector

BNC connector has 75 ohm for serial digital interface and 50 ohm for RF communication

HD-SDI Cable uses 75ohm Cable therefore you may use 75ohm BNC connector. In case of using 50ohm, it may not work due to too much reflexibility.

<Note> Problems of using 50ohm connector of 75ohm cable

. It increases the transmission loss on circuit.

. The circuit has an error and turns unstable.

. The machine or parts of circuit (IC, transistor , resistor etc) is heated.

How to make BNC cable

- Cable is produced by using Tool kit offered by Connector company.

How to make BNC cable

<Note> connector work is connecting cable’s internal connector to connector.

Brazing method is common one from long times ago. Brazing increases resistor value because the metal is added when brazing. Therefore, you should use a connector with pressing method. Also, pressing degree should be well measured according to the specification recommended by the manufacturer.

Appendix 5. Cautions for installing HD-SDI  Cable

- HD SDI is a Single link method.

When extending, you should use HD SDI repeater.

You should not try to extend it by using lines or other connectors.

You should use HD-SDI distributor for distributing the same image.

You may not connect two cables by brazing or distribute by using Goal post

- You should not use cable with power lines for HD SDI Cable.

- BNC cable + DC power cable

- Appendix 6.  EIO Hardware

6.1  EIO 0404

. Major Connectors of the EIO-0404 Card

① 20 pin box connector (J3) for connecting the sensor/control capture card

② 20 pin box connector (J5) for connecting the sensor/control extension card

③ Switch for selecting sensor type (SW2) (refer to Appendix 1)

④ Switch for board ID(SW1)

⑤ 4pin connector for connecting RS-485/422 Devices (J1) (refer to Appendix 2)

⑥ DI 1-4 terminal block for connecting sensor input signal (J4)

An external sensor device is connected to J4.  The terminal port from the top to the bottom is 1, 2, 3, and 4 and the lowest terminal port is used as a common port.

⑦ DO 1-4 terminal block for connecting control output signal (J6)

This connector is used to send out DO signals. The terminal port from the top to the bottom is 1, 2, 3, and 4 and the lowest terminal port is used as a common port

⑧ 4 pin I2C connector for connecting sensor/control extension card (J7)

This connector is used when connecting an EIO card with a 4 pin cable. Only EIO card series have this connector.

⑨ 4 pin I2C connector for connecting sensor/control capture card (J8)

This connector is exactly same as a J7 connector. It is a spare connector for additional connection.

⑩ 5 pin USB connector for PC (J9)

It is connected to the internal USB port on the motherboard.

- 6.2  EIO 1200

Major Connectors of the EIO-1200 Card

① 20 pin box connector(J4) for connecting the sensor/control capture card

② 20 pin box connector(J6) for connecting the sensor/control extension card

③ Switch for selecting sensor type(SW2) (refer to appendix 1)

④ DI 1-4 terminal block for sensor input signal connection (J3)

⑤ DI 5-8 terminal block for sensor input signal connection (J5)

⑥ DI 9-12 terminal block for sensor input signal connection (J9)

⑦ 4 pin connector for connecting sensor/control extension card(J8)

⑧ 4 pin connector for connecting sensor/control capture card(J7)

⑨ Switch for board ID(SW1)

Appendix 1.  Switch for selecting sensor type(SW2)

► DI interface type: Relay type/Voltage type

► Input operating range (if turned ON)

|  | Min | Typ | Max |
| --- | --- | --- | --- |
| Voltage Type (DC Voltage) | 3V | 5V | 12V |
| Relay Type (Conductor resistance) | 0Ω | - | 170 Ω |

selected on EIO card.

| DI Interface Type | RELAY | VOLTAGE |
| --- | --- | --- |
| DI_TYPE value | 1 | 0 |
| Switch(SW2)*1 | ON,ON | OFF,OFF |
| External Action | Metallic contact | Voltage level |
| Isolation | X | O |

► DI Select Switch(SW2)

| SWITCH(SW2) | TYPE |
| --- | --- |
| ON,ON | Relay |
| OFF,OFF | Voltage |

- . DI Select switch and its status table

Appendix 2.  4pin connector for connecting RS-485/422 Devices (J1)

► Standard RS-485, RS-422

► Input Operating Range Min: -7V, Max +12VDC

► Able to connect up to 128 devices

► RS-485/422 Pin Assignment (J1)

| J1 Pin Number | RS-485 | RS-422 |
| --- | --- | --- |
| 1 | TRX+ | TX+ |
| 2 | TRX- | TX- |
| 3 | X | RX+ |
| 4 | X | RX- |

.