Probe Inc.

16 MARCH 2011

SDK Description

Ver.0.0.2e

Orders

Document History	4

1. Overview	5

2. SDK Connection Process	5

3. Interface	7

4. Check Point to FAQ	11

- Document History

| Date | Note | Author | Revision |
| --- | --- | --- | --- |
| 2011-01-28 | -Initial Version | Lee Sang Min | 0.0.1 |
| 2011-03-16 | -Comment for SDK file | Alex Oh | 0.0.2 |

1. Overview

- This document is for the brief explanation of SDK skim to be used with source program packaged together. this document can be referred with BASIC NETWORK PROTOCL for PNP which is detailed with PROBE Network Protocol.

# 2. SDK Use

Source folders in SDK for respective use,

- SDK Source Folder: Full source file supporting the direct compile and debugging

*Compile should be made to execute the source

2) SDK Sample Folder: Executable file (ProbeDemo) with corresponding “ *.dll” files.

# 3. SDK Connection Process

1. _PROBENET_INIT(); 2. HANDLE _PROBENET_CONNECTION_INIT(CONNECTION_INFO *lpConnectionInfo, DWORD dwMaskCreatingSocks, DWORD dwProductType=11); 3. _PROBENET_CONNECTION_SETCTRLCALLFUNC( m_hConnection, CLiveChannel::_ON_RECV_CTRL_DATA, CLiveChannel::_ON_ERROR_CTRL, (HANDLE)this); _PROBENET_CONNECTION_SETSTREAMCALLFUNC( m_hConnection, CLiveChannel::_ON_RECV_VIDEO_DATA, CLiveChannel::_ON_RECV_AUDIO_SPEAKER_DATA, CLiveChannel::_ON_RECV_VIDEO_CTRL_DATA, CLiveChannel::_ON_RECV_AUDIO_SPEAKER_CTRL_DATA, CLiveChannel::_ON_ERROR_VIDEO, CLiveChannel::_ON_ERROR_AUDIO_SPEAKER, (HANDLE)this); _PROBENET_CONNECTION_SETAUDIOSENDERCALLFUNC( m_hConnection, CLiveChannel::_ON_RECV_AUDIO_MIC_CTRL_DATA, CLiveChannel::_ON_ERROR_AUDIO_MIC, (HANDLE)this); 4.1 Actual connection is established once “CNS" will be received on Control Receive after requesting with _PROBENET_CONNECTION_CONNECT( m_hConnection, _CONTROL_SOCK);* _CONTROL_SOCK. After this establishment, next step can be tried to t with _VIDEO_SOCK ¦ _AUDUIO_SOCKET. * void _PROBENET_CONNECTION_SETCHANNEL(HANDLE handle, int nChannel); 4-2._PROBENET_CONNECTION_CONNECT( m_hConnection, _VIDEO_SOCK ); 4-3._PROBENET_CONNECTION_CONNECT( m_hConnection, _AUDIO_SOCK ); 5. void _PROBENET_CONNECTION_DISCONNECTALL( HANDLE handle ); 6. void _PROBENET_CONNECTION_END(HANDLE handle); 7. void _PROBENET_END();

# 3. Interface

 Registration for SDK use Return Value; Function. Void; _PROBENET_INIT. Return Value; Function. Void; _PROBENET_END.

|  SDK Initialization Return Value; Function. HANDLE; _PROBENET_CONNECTION_INIT. Parameter Index; Type; Name; Description. 1; CONNECTION_INFO *; lpConnectionInfo; Connection info. 2; DWORD; dwMaskCreatingSocks; Control, Video, Audio( Speaker ). 3; DWORD; dwProductType=11; ( DM365 Product = 11 ) *always setup with 11. Return Value; Function. Void; _PROBENET_CONNECTION_END. Parameter Index; Type; Name; Description. 1; HANDLE; handle; Connected handle. |  |
| --- | --- |
|  Call-Back Function Registration ( Control ) Return Value; Function. void; _PROBENET_CONNECTION_SETCTRLCALLFUNC. Parameter Index; Type; Name; description. 1; HANDLE; handle; SDK Initializing Handle. 2; LPSOCKETFUNC_ON_RECV_CTRL_DATA; OnRecvCtrlMsg; Callback for CMD receive. 3; LPSOCKETFUNC_ON_ERROR; OnError; Error Callback for Error receive. 4; HANDLE; hCallback; Present Handle( this ). |

 Call-Back Function Registration (Video, Audio ( Speaker ) ) Return Value; Function. void; _PROBENET_CONNECTION_SETSTREAMCALLFUNC. Parameter Index; Type; Name; description. 1; HANDEL; handle; SDK Initializing Handle. 2; LPSOCKETFUNC_ON_RECV_VIDEO_DATA; OnRecvVideo; Callback for Data command receive. 3; LPSOCKETFUNC_ON_RECV_AUDIO_DATA; OnRecvAudio; Callback for Data command receive. 4; LPSOCKETFUNC_ON_RECV_CTRL_DATA; OnRecvVideoMsg; Callback for Data command receive. 5; LPSOCKETFUNC_ON_RECV_CTRL_DATA; OnRecvAudioMsg; Callback for Data command receive. 6; LPSOCKETFUNC_ON_ERROR; OnErrorVideo; Callback for Video Error receive. 7; LPSOCKETFUNC_ON_ERROR; OnErrorAudio; Callback for Speaker Error receive. 8; HANDLE; hCallback; Present Handle( this ).

 Call-Back Function Registration ( Audio ( Microphone ) ) Return Value; Function. void; _PROBENET_CONNECTION_SETAUDIOSENDERCALLFUNC. Parameter Index; Type; Name; description. 1; HANDLE; handle; SDK Initializing Handle. 2; LPSOCKETFUNC_ON_RECV_CTRL_DATA; OnRecvCtrlMsg; Callback for CMD receive. 3; LPSOCKETFUNC_ON_ERROR; OnError; Callback for Error Receive. 4; HANDLE; hCallback; Present Handle( this).

 Connect, Disconnect ( Control, Video, Audio( Speaker ), Audio( Microphone ) ) Return Value; Function. void; _PROBENET_CONNECTION_CONNECT _PROBENET_CONNECTION_DISCONNECT. Parameter Index; Type; Name; Description. 1; HANDLE; handle; SDK Initializing Handle. 2; DWORD; dwMaskCreatingSocks; Control, Video, Audio( Speaker, Microphone ). Return Value; Function. void; _PROBENET_CONNECTION_DISCONNECTALL. Parameter Index; Type; Name; Description. 1; HANDEL; handle; SDK Initializing Handle.

 Control Command Return Value; Function. void; _PROBENET_CONNECTION_SETCHANNEL. Parameter Index; Type; Name; description. 1; HANDEL; handle; SDK Initializing Handle. 2; Int; nChannel; Channel to be changed. Return Value; Function. int; _PROBENET_CONNECTION_SENDCOMMAND. Parameter Index; Type; Name; description. 1; HANDLE; handle; SDK Initializing Handle. 2; char *; szCmd; Command. 3; BYTE *; lpParam; Command Parameter Info. 4; DWORD; dwParamSize; Command Parameter Size. Return Value; Function. int; _PROBENET_CONNECTION_SENDAUDIOPACK. Parameter Index; Type; Name; Description. 1; HANDEL; handle; SDK Initializing Handle. 2; AUDIO_DATA_PACK *; lpData; Speaker information.

 Encoder Setup Return Value; Function. BOOL; _PROBENET_CONNECTION_GETENCODESETUPINFO. Parameter Index; Type; Name; description. 1; HANDLE; handle; SDK Initializing Handle. 2; Int; nChannel; Channel. 3; ENCODE_SETUP_INFO *; lpData; Encoder Setup info. 4; BOOL; bRefreshData=TRUE; Encoder Setup Update. Return Value; Function. int; _PROBENET_CONNECTION_SETENCODESETUPINFO. Parameter Index; Type; Name; Description. 1; HANDEL; handle; SDK Initializing Handle. 2; ENCODE_SETUP_INFO *; lpData; Encoder Setup info.

- 4. Check Point to FAQ

 it takes “1” second to create the random code(value) in CNS command so it’s recommended not to send the connection request to the camera within 1 second  it’s one of specific skim from IOCP to create threads internally with same number like the number of CPU core for the improvement of streaming performance  AlarmIn : Only “Alarm ON” status, Camera will send the alarm signal(alarm ON event) to SDK so, SDK will send the same “ON” event notification to Client(SW). in the case of “Alarm Off”, SDK detects its change after 5 seconds if there is no further “Alarm ON” notification from camera and sends the “off event” to Client(SW)  Connection status check : “STA” event can be used for this purpose : Server --(“STA”)--> Client --(“STA”)--> Client ( every 10 seconds )  In order to get the name of camera name and MAC address, SW can get the information from camera through the channel information from “RSD” command. (_PROBENET_CONNECTION_SETCHANNEL() )  Audio setting value [ Audio( Speaker & Mic) ] * Set WAVEFORMATEX wfx; wfx.wFormatTag = WAVE_FORMAT_PCM; // 1 wfx.nChannels = AT_MONO; // 1 wfx.nSamplesPerSec = AUDIO_SAMPLE8K_PER_SEC; // 8,000 wfx.wBitsPerSample = AUDIO_BITS_FOR_SAMPLE; // 16 wfx.nBlockAlign = (wfx.wBitsPerSample / 8) * wfx.nChannels; wfx.nAvgBytesPerSec = wfx.nSamplesPerSec * wfx.nChannels * wfx.nBlockAlign; wfx.cbSize = 0;  GOS size change with Frame-Rate setting * The change of GOP value will be not applied if frameRate value will be same without change * If value of FrameRate is changed, then following table will be applied. FrameRate; Changed GOP; GOP (H.264); GOP (MJPEG); GOP (MPEG4). <10; Any value; Same value with FrameRate (follow FrameRate). =,>10; <10; 15 (always). =,>10; =,>10; Follow GOP setup(changed) value accordingly.. No change; 1~30; No change; No hcange; No change.